This code aims at identifying sea-ice floes, sea-ice types, and presenting a color map of the ice floes and brash pieces based on sizes, and the corresponding ice floe size distribution histogram.

The GVF snake algorithm used in this code is an open source code, which is available on: http://www.iacl.ece.jhu.edu/static/gvf/ .

This code is only available for academic non-commercial use. If you are using this code, please give a reference to:

Zhang, Q. and R. Skjetne, “Image processing for identification of sea-ice floes and the floe size distributions.” IEEE Transactions on Geoscience and Remote Sensing, Vol. 53, No. 5, pp. 2913-2924, 2015.

Function: seaice_kmean_GVF_forenhancement

Sea-ice image segmentation program. This program is suit for non-ridged ice floe in the marginal ice zone, and managed ice resulting from offshore operations in sea-ice.

Example:

[seg, bk] = seaice_kmean_GVF_forenhancement( I, kms0, sigma, GradientOn, GVFOn, Num, mu, iter, alpha, beta, gamma, kappa, Dmin, Dmax, N, Ra_min, Ra, Rc, Rl, se, timer);

Input:

I: the input ice image (the same as in GVF_distance).

% parameters for image

kms0: the number of clusters for k-means clustering to detect all the ice.

Ra_min: the minimum ice piece area, the ice piece area less than Ra_min is considered as noise (the same as in GVF_distance).

Ra: the maximum ice piece area, the ice piece area larger than Ra is considered as under-segmentation, another turn of segmentation is required (the same as in GVF_distance).

Rc: convexity threshold, the ice piece convexity less than Rc is considered as under-segmentation, another turn of segmentation is required (the same as in GVF_distance).

Rl: threshold ratio between length and width, ratio between length and width larger than Rl is considered as under-segmentation, another turn of segmentation is required (the same as in GVF_distance).

timer: limit the maximum iterations of segmentation (the same as in GVF_distance).

se: morphology structure element, for combining the nearby local maxima (the same as in GVF_distance).

% parameters for GVF Snake, MOST TUNE!

Num: number of GVF iterations, the most important, tune parameter. For weak boundaries, large ice floe, high resolution image, Num should be increased (the same as in GVF_distance).

iter: number of Snake iterations, adjust it according to the over- and under-segmentation result (the same as in GVF_distance).

N: iter of deformation, adjust it according to the over- and under-segmentation result (the same as in GVF_distance).

(The computation time of the algorithm will increase when increasing Num, iter, N.)

(Do not tune parameters which are not mentioned above.)

Output:

seg: the segmented sea-ice image.

bk: binary non-segmented ice detection image.

Function: ice_shape_enhancement

Enhance the ice floe (brash) shape; separate the sea-ice image into four layers: ice floe, brash ice, slush, and water; calculate the floe and brash size.

Example:

[out, index_floe, ice_floe, index_brash, brash_ice, index_slush, index_water, index_residue, coverage] = ice_shape_enhancement(bk, seg, min_floe, min_brash, se_th);

Input:

bk: binary non-segmented ice detection image (output of seaice_kmean_GVF_forenhancement).

seg: the segmented sea-ice image (output of seaice_kmean_GVF_forenhancement).

% parameters for shape enhancement

min_floe: minimum floe area, to identify ice floe from brash.

min_brash: minimum brash area.

se_th: threshold for adaptive morphology structure element.

radium of morphology structure element=1,  ice piece area≤se_th2,  ice piece area>se_th

Output:

Out: morphology cleaned ice pieces (ice floe & brash ice).

index_floe: layer of ice floe.

ice_floe: information about each identified ice floe.

index_brash: layer of brash ice.

brash_ice: information about each identified brash ice.

index_slush: layer of slush ice.

index_water: layer of water.

index_floe: layer of ice floe.

index_residue:   layer of residue.

coverage: coverage of each layer.

Function: sea_ice_model

Polygon-fit for each sea-ice floe, and disk-fit for each brash ice.

Example:

[floe, brash] = sea_ice_model( ice_floe, brash_ice, index_floe );

Input:

ice_floe: structure data of identified sea-ice floes (output of ice_shape_enhancement).

brash_ice: structure data of identified brash ice (output of ice_shape_enhancement).

img: segmented (identified) binary image

Output:

floe: structure data, includes: Vertices, Center, Area, Perimeter, and overlapping floes.

brash: structure data, includes: Vertices, Center, Area, Perimeter, and overlapping brash.

An example of sea-ice image segmentation is given in the file: sea_ice_demo.m.