This code aims at identifying model-ice floes, presenting a color map of the ice floes based on sizes with the corresponding ice floe size distribution histogram, and rectangularizing the identified ice floes.

The GVF snake algorithm used in this code is an open source code, which is available on: http://www.iacl.ece.jhu.edu/static/gvf/ .

This code is only available for academic non-commercial use. If you are using this code, please give a reference to:

Zhang, Q., R. Skjetne, I. Metrikin, and S. Løset, “Image Processing for Ice Floe Analyses in Broken-ice Model Testing.” Cold Regions Science and Technology, Vol. 111, pp. 27-38, 2015.

Function: GVF_distance

Model-ice image segmentation program.

Example:

Bw = GVF_distance(I, sigma, GradientOn, GVFOn, Num, mu, iter, alpha, beta, gamma, kappa, Dmin, Dmax, N, Ra_min, Ra, Rc, Rl, se, timer);

Input:

I: the input ice image.

% parameters for image

Ra_min: the minimum ice floe (piece) area, the ice piece area less than Ra_min is considered as noise.

Ra: the maximum ice floe (piece) area, the ice piece area larger than Ra is considered as under-segmentation, another turn of segmentation is required.

Rc: convexity threshold, the ice piece convexity less than Rc is considered as under-segmentation, another turn of segmentation is required.

Rl: threshold ratio between length and width, ratio between length and width larger than Rl is considered as under-segmentation, another turn of segmentation is required.

timer: limit the maximum iterations of segmentation.

se: morphology structure element, for combining the nearby local maxima.

(The criteria in this algorithm were particular designed for square-shape-floe, but it can be also used for other conditions by tune the parameter of Rc and Rl.)

% parameters for GVF Snake, MOST TUNE!

Num: number of GVF iterations, the most important, tune parameter. For weak boundaries, large ice floe, high resolution image, Num should be increased.

iter: number of Snake iterations, adjust it according to the over- and under-segmentation result.

N: iter of deformation, adjust it according to the over- and under-segmentation result.

(The computation time of the algorithm will increase when increasing Num, iter, N.)

(Do not tune parameters which are not mentioned above.)

Output:

bw: a segmented binary image.

Function: rect

Ice floe rectangularization.

Example:

S = rect(Bw);

The input (Bw) is the segmented model-ice image(the output of function GVF_distance).

The output (S) is a structure that includes vertices, position, size of each rectangularized ice floe.

Function: model_ice_model

Example:

s_model = model_ice_model( S, img, k1, k2 )

Input:

S: structure data of rectangularized floes (output of function rect).

img: segmented binary image.

k1: minimum length/width radio of model ice floe.

k2: maximum length/width radio of model ice floe.

An example of model-ice image segmentation is given in the file: model_ice_demo.m